BIRLA INSTITUTE OF TECHNOLOGY & SCIENCE, PILANI

Work Integrated Learning Programmes Division

AIML ZG535— Machine Learning on Edge

MINI-PROJECT

LogiEdge: Intelligent Edge AI Platform for Cold-Chain Logistics

Continuous Evaluation Component – EC-I | 30 Marks | Duration: 10 Weeks

| Field | Details |
| --- | --- |
| Instructor | Pravin Yashwant Pawar |
| Programme | AIML ZG535 — Machine Learning on Edge |
| Total Marks | 30 marks (Continuous Evaluation Component – EC I ) |
| Duration | 10 Weeks (approximately 2 months) |
| Domain | Logistics — Cold-Chain Fleet Monitoring |
| Modules Covered | Modules 1 through 6 ¦ Sessions 1 through 12 |
| Submission Mode | GitHub Repository + Demo Video + PDF Report via LMS |
| Final Submission | Check course page |

# 1.  Background and Business Context

FreightBridge Logistics Pvt. Ltd. is a mid-sized third-party logistics company headquartered in Pune, operating across Maharashtra, Karnataka, and Andhra Pradesh. The company specialises in three business verticals:

- Cold-chain pharmaceuticals: 85 refrigerated trucks transporting temperature-sensitive medicines, vaccines, and biologics to district hospitals, PHCs, and retail pharmacies

- Perishable food distribution: 60 trucks delivering dairy, seafood, and packaged produce from processing plants to supermarket chains

- Industrial spares: 120 standard trucks delivering auto components and electronic goods between factories, warehouses, and dealers

FreightBridge currently operates with manual temperature log books inside refrigerated trucks (filled by drivers every 2 hours), no predictive maintenance capability, and a cloud-based GPS tracking system that loses connectivity in rural Maharashtra and Andhra Pradesh hill regions. The company has experienced three major incidents in the past 18 months:

- A ₹28 lakh vaccine spoilage event when a refrigeration unit failed undetected on a Nashik–Aurangabad route

- A rejection of a pharmaceutical shipment at a hospital due to incomplete temperature chain-of-custody documentation

- Two breakdowns of trucks mid-route due to engine anomalies that were detectable in sensor data three days earlier

The company's operations director has approved a pilot Edge AI programme. You are the Edge AI engineer appointed to design, build, and validate the core intelligence layer of this system. The pilot covers cold-chain truck monitoring only — the 85 refrigerated trucks — with the explicit expectation that a successful pilot will be scaled to all 265 vehicles.

# 2.  Problem Statement

Design and implement LogiEdge — an Edge AI pipeline for real-time cold-chain monitoring on FreightBridge's refrigerated trucks. The system must:

- Continuously monitor cargo temperature, refrigeration unit vibration, and door open/close events

- Classify the cargo compartment state into three operational classes in real time on the truck's edge device

- Alert the operations centre immediately when a violation or anomaly is detected, without depending on continuous cellular connectivity

- Demonstrate readiness for fleet-scale deployment through hardware justification, model optimisation, and an MLOps monitoring capability

The three classification targets are:

| Class | Label | Condition |
| --- | --- | --- |
| 0 | Normal | Temperature within ±1°C of setpoint; no vibration anomaly; refrigeration unit operating correctly |
| 1 | Warning | Temperature drifting (1–3°C outside setpoint) OR early refrigeration anomaly detected; intervention needed within 2 hours |
| 2 | Critical | Temperature breach (>3°C outside setpoint) OR refrigeration unit failure signature; immediate escalation required |

⚠ IMPORTANT:  The system must operate fully offline during cellular connectivity gaps and synchronise alert logs with the operations centre when cellular coverage returns. A cloud-dependent architecture is not acceptable for this deployment.

# 3.  Scope and Module Coverage

This project spans Modules 1 through 6 (Sessions 1 through 12). Each module contributes a distinct and assessable component of the LogiEdge system.

| Module | Sessions | LogiEdge Component |
| --- | --- | --- |
| 1 — Foundations of Edge AI | 1 – 2 | Deployment justification and system architecture design |
| 2 — Edge Hardware & Systems | 3 – 4 | Hardware selection, Constraint Triangle, Arithmetic Intensity, Roofline model |
| 3 — Edge Data & IoT Integration | 5 – 6 | Sensor simulator, MQTT architecture, preprocessing pipeline, data fusion |
| 4 — Model Lifecycle & Deployment | 7 – 8 | Model training, TFLite conversion, Docker containerisation, OTA demo |
| 5 — Edge MLOps & Management | 9 – 10 | PSI drift monitoring, Ansible deployment, OTA strategy selection |
| 6 — Model Optimisation Techniques | 11 – 12 | Three model variants, benchmarking, Pareto frontier analysis |

# 4.  Technical Requirements by Component

## Component A — System Architecture and Deployment Justification [Module 1]

Submit this section’s responses as a part Final Report Section in PDF format only.

### Task A1 — Constraint Analysis (350–500 words)

Write a structured analysis addressing all four Edge AI constraints for the FreightBridge cold-chain deployment:

- Latency: A refrigeration unit failure can raise cargo temperature by 1°C per minute. The system must detect and alert within 90 seconds of a fault signature appearing in sensor data. Quantify whether cloud inference is feasible given India's rural cellular round-trip latency profile.

- Bandwidth: Each truck generates temperature readings at 1 Hz, vibration at 500 Hz (3-axis), and door events as discrete events. Calculate the raw data volume per truck per day. Calculate the transmission cost at ₹0.10/MB. Compare against the edge-processed alert volume.

- Connectivity: FreightBridge's Nashik–Aurangabad route loses cellular signal for 35–90 minutes at seven documented locations. Describe what happens to a cloud-only system during these gaps and how your edge architecture handles them.

- Privacy: FreightBridge's pharmaceutical clients require proof that cargo condition data cannot be accessed by unauthorised third parties. Explain how on-device inference supports this contractually.

### Task A2 — System Architecture Diagram

Draw a complete system architecture diagram showing: the truck's edge node, sensor connections, local MQTT broker, inference pipeline, local alert log, cellular uplink, and operations centre backend. Format is not assessed — clarity is. Include in final Report.

## Component B — Hardware Selection and Justification [Module 2]

### Task B1 — Constraint Triangle Application

Three hardware options are under consideration for the on-truck edge node:

| Option | Hardware | India Price | TDP |
| --- | --- | --- | --- |
| 1 | Raspberry Pi 5 (8 GB) + AI HAT+ (13 TOPS Hailo-8L) | ~₹15,000/truck | 7.5W |
| 2 | Jetson Orin Nano Super Developer Kit (67 TOPS) | ~₹45,000/truck | 15W moderate load |
| 3 | STM32H7-based custom MCU with sensor ICs | ~₹3,500/truck | 0.4W |

Apply the Constraint Triangle to all three options. State the dominant constraint vertex for the cold-chain deployment. Argue for one option and against the other two. Your argument must reference the 90-second latency requirement, the 10W AI power budget (12V truck supply via DC-DC converter), and fleet-scale unit cost (85 trucks pilot, 265 at full scale).

### Task B2 — Arithmetic Intensity and Roofline Analysis

Your trained model performs approximately 45 MFLOPs per inference and accesses approximately 18 MB of data (weights + activations) per inference. The Raspberry Pi 5 CPU has approximately 16 GFLOP/s (NEON SIMD) and 12 GB/s LPDDR4X bandwidth. Calculate the Arithmetic Intensity, find the ridge point, classify the model as memory-bandwidth bound or compute-bound, and state what optimisation would improve latency based on the Roofline model.

## Component C — Sensor Pipeline and MQTT Architecture [Module 3]

### Task C1 — Sensor Simulator

Build a Python sensor simulator that generates realistic cold-chain truck sensor data with three streams:

| Stream | Frequency | Normal Parameters | Anomaly Mode |
| --- | --- | --- | --- |
| temperature | 1 Hz | N(4.0°C, 0.3) — setpoint 4°C | temp_drift: +0.08°C per reading (linear drift) |
| vibration_rms | 0.5 Hz | N(0.45g, 0.05) — compressor RMS | vibration: step to N(1.2g, 0.15) — bearing wear |
| door_event | Discrete event | 'OPEN' or 'CLOSE' with timestamp | combined: both anomalies simultaneously |

The simulator must accept a CLI flag --anomaly {none|temp_drift|vibration|combined}. All streams must publish to a local Mosquitto broker on localhost.

### Task C2 — Preprocessing Pipeline

Implement the complete preprocessing pipeline in the following sequence:

- Filtering: 5-sample moving average on temperature and vibration streams

- Feature extraction per 30-second sliding window (step = 10 seconds): temperature mean, temperature standard deviation, temperature rate-of-change (°C/min), vibration RMS, vibration peak, vibration kurtosis — 6-value feature vector

- Normalisation: compute mean and std of each feature from 10 minutes of clean Normal-class output; save as training_stats.npy; load at runtime — never recompute from live data

- Mandatory experiment: run inference with correct stats, then with stats shifted by 3σ; report accuracy changes in Phase 2 Report

### Task C3 — Data Fusion Justification

Implement feature-level fusion: extract features from each sensor independently, then concatenate into one 6-value joint feature vector before passing to the model. In your Final Report, justify this fusion level choice against data-level and decision-level alternatives for this specific application.

## Component D — Model Training, Conversion, and Docker Deployment [Module 4]

### Task D1 — Dataset Generation and Model Training

Generate a labelled training dataset by running the simulator in each mode for the durations below:

| Class | Label | Simulator Mode | Duration | Approx. Samples |
| --- | --- | --- | --- | --- |
| 0 | Normal | --anomaly none | 20 minutes | ~120 windows |
| 1 | Warning | --anomaly temp_drift | 15 minutes | ~90 windows |
| 2 | Critical | --anomaly combined | 15 minutes | ~90 windows |

Train a classification model (recommended: 2-hidden-layer MLP with 32 and 16 units, ReLU activation). Accuracy on a held-out validation set (20% split) must exceed 88% — if it does not, revisit feature extraction or architecture before proceeding.

⚠ IMPORTANT:  Validation accuracy below 88% means the model is not fit for pharmaceutical cold-chain monitoring. The project cannot be graded below this threshold for the model training component.

### Task D2 — Docker Containerisation and OTA Demo

Package the inference service (preprocessing → model inference → MQTT result publish) into a Docker container:

- Base image: python:3.11-slim

- All pip install layers must appear before the COPY model.tflite . instruction

- Container must accept environment variable MODEL_PATH to switch model variants without rebuild

- Container must publish to MQTT topic: logibridge/trucks/{truck_id}/inference

Perform the OTA layer-cache demonstration: change only the model file, rebuild, and show that only the model layer is rebuilt. Calculate and state the bandwidth saving for 85 trucks in your demo video.

### Task D3 — The 10-Stage Pipeline Mapping

In Final Report Section 3, map LogiEdge explicitly to all 10 stages of the Edge ML pipeline from Module 4. For each stage, write 1–2 sentences describing what happens in your specific system — not a generic description.

## Component E — Edge MLOps: Monitoring and Deployment Management [Module 5]

### Task E1 — PSI Drift Monitoring

Implement Population Stability Index (PSI) monitoring on the model's output confidence score distribution.

- Reference distribution: run inference on 300 clean Normal-class windows; compute confidence score distribution across 4 bins ([0, 0.25), [0.25, 0.50), [0.50, 0.75), [0.75, 1.0]); save as reference_dist.json

- Monitoring: compute PSI on rolling window of last 100 inferences every 60 seconds; print current PSI value

- Alert when PSI > 0.25: print [LOGIBRIDGE DRIFT ALERT] PSI={value:.3f}

- Drift injection: activate --anomaly combined mid-run; PSI must cross 0.25 within 5 minutes

- Recovery: restore clean data; show PSI returning below 0.10 — all visible in demo video

### Task E2 — Ansible Deployment Playbook

Write logibridge_deploy.yml with exactly 7 tasks:

- Create /opt/logibridge directory if not present

- Copy new model file to /opt/logibridge/model.tflite

- Copy updated reference_dist.json

- Stop the running inference container if running

- Pull updated container image from local Docker registry

- Start inference container with environment variables

- Wait 15 seconds and verify container is running

Run the playbook twice without changes between runs. Show in terminal that the second run reports changed=0. Both runs must appear in the demo video.

### Task E3 — OTA Strategy Selection

FreightBridge updates truck models every 6 weeks. Model file is 280 KB (INT8 TFLite). Trucks use M2M SIM at ₹0.10/MB. 85 trucks in the pilot fleet. Evaluate three OTA strategies: full replacement, canary (10 trucks first), shadow mode. Calculate bandwidth cost per update cycle for each. Recommend one strategy and argue against the other two, referencing cold-chain safety-criticality and rural connectivity characteristics.

## Component F — Model Optimisation [Module 6]

### Task F1 — Three Model Variants

| Variant | Method | Tools Required |
| --- | --- | --- |
| M1 — FP32 Baseline | Standard training; Keras SavedModel or PyTorch | tf.keras or PyTorch |
| M2 — PTQ INT8 | Post-Training Quantisation, Full INT8, ≥200 calibration samples | tf.lite.TFLiteConverter with DEFAULT optimisation + representative dataset |
| M3 — Structured Pruning + PTQ INT8 | 35% structured filter pruning (PolynomialDecay schedule) followed by Full INT8 PTQ | tensorflow_model_optimization + TFLiteConverter |

### Task F2 — Five-Metric Benchmarking

Benchmark all three variants using Lab 2 methodology. Measure all five metrics for each variant:

- Mean inference latency (ms) — 200 runs after 10 warm-up runs excluded

- p95 inference latency (ms)

- Model file size (KB)

- Classification accuracy on held-out validation set (%)

- Energy per inference (mJ) — using E = P × t with psutil CPU% and laptop TDP estimate

### Task F3 — Deployment Recommendation

Answer the operations director's question 'Which model should we deploy to the 85 trucks?' with structured reasoning referencing: the 90-second SLA translated to per-inference latency budget; hardware Flash and SRAM constraints for your chosen device; and Class 2 (Critical) recall on the recommended variant — which must exceed 95%.

# 5.  Deliverables

⚠ IMPORTANT:  All three deliverables are mandatory. A submission missing any one of the three will be capped at 18 marks regardless of quality of the other two.

## Deliverable 1 — GitHub Repository

Submit a private GitHub repository URL with the instructor added as a collaborator. Suggested repository structure:

logibridge/

├── README.md

├── scenario_architecture/

│   ├── constraint_analysis.md

│   └── system_architecture.png

├── hardware/

│   └── hardware_justification.md

├── data_pipeline/

│   ├── simulator.py

│   ├── preprocessing.py

│   ├── training_stats.npy

│   └── mqtt_architecture.md

├── training/

│   ├── generate_dataset.py

│   ├── train_model.py

│   ├── convert_ptq.py

│   ├── prune_quantise.py

│   └── models/

├── inference/

│   ├── Dockerfile

│   ├── inference_service.py

│   └── model.tflite

├── monitoring/

│   ├── drift_monitor.py

│   └── reference_dist.json

├── deployment/

│   └── logibridge_deploy.yml

├── optimisation/

│   ├── benchmark.py

│   └── results/

│       ├── benchmark_results.csv

│       └── pareto_chart.png

└── reports/

├── phase1_report.pdf

├── phase2_report.pdf

└── final_report.pdf

## Deliverable 2 — Demo Video (15–20 minutes)

Screen-recorded video submitted as MP4 or YouTube/Google Drive link (set to 'anyone with link'). Test the link from incognito before submitting.

## Deliverable 3 — Final Report (2,500–3,000 words)

Single PDF report. Word count excludes figures, tables, and references.

| Report Section | Words | Content |
| --- | --- | --- |
| 1 — FreightBridge Deployment Context | 400–500 | Four Edge AI constraints with numbers; hardware selection with Constraint Triangle; system architecture diagram; Arithmetic Intensity result and Roofline classification |
| 2 — Sensor Pipeline and MQTT Design | 500–600 | Full MQTT topic tree (as figure); QoS justification per topic; preprocessing design rationale; normalisation experiment results table; data fusion choice justified against alternatives |
| 3 — Model Deployment and Pipeline Mapping | 400–500 | 10-stage pipeline mapped to LogiEdge components; Docker layer-cache result; bandwidth saving for 85 trucks; OTA strategy recommendation with financial justification |
| 4 — Optimisation and Pareto Analysis | 400–500 | Benchmarking results table (15 cells); Pareto chart with annotation; Class 2 recall for recommended variant; deployment recommendation with SLA evidence |
| 5 — MLOps Monitoring and Reflection | 300–400 | PSI values before/during/after injection; drift type classification; Ansible idempotency result; one genuine technical difficulty; one architectural change you would make |

# 6.  Evaluation Rubric — 30 Marks

| Section | Component Assessed | Marks |
| --- | --- | --- |
| 1 | System Architecture and Hardware Justification | 7 |
| 2 | Sensor Pipeline and MQTT Architecture | 7 |
| 3 | Model Training, Conversion, and Docker Deployment | 6 |
| 4 | Model Optimisation and Pareto Analysis | 5 |
| 5 | MLOps: Drift Monitoring and Ansible Deployment | 3 |
| 6 | Final Report Quality | 2 |
|  | TOTAL | 30 |

# 7.  Submission Requirements

## Final Submission — check course page for exact date and time

Submit via the LMS assignment portal under 'Mini-Project — LogiEdge Final Submission'. Your submission must contain exactly three items:

| Item | Format | Naming / Access Requirement |
| --- | --- | --- |
| GitHub Repository URL | Text in submission comment field and report | Instructor must be added as collaborator before submission date |
| Demo Video | YouTube unlisted or Google Drive link in demo/demo_video_link.txt in repository and in report | Set to 'anyone with the link'; test from incognito browser before submitting |
| Final Report PDF | Upload to LMS portal | Filename: GROUPNO_LogiEdge_Final.pdf |

⚠ IMPORTANT:  Late submission penalty: 2 marks per calendar day, maximum deduction 10 marks. Submissions more than 5 days late will not be accepted.

# 8.  Academic Integrity

The demo video is the primary integrity mechanism. A working pipeline running live in real time cannot be fabricated by an AI assistant or borrowed from a classmate — it requires you to have actually built and debugged the system. The following constitute academic misconduct:

- Submitting a video that was recorded by or shows the system of another person

- Submitting code, you cannot explain under follow-up questioning

- Submitting code functionally identical to another student's with superficial edits — code similarity tools will be applied to all submissions

- Generating the Final Report exclusively through an AI writing tool without substantial original analysis and measured data

You are encouraged to use course lab scripts as starting points, consult official documentation (TFLite, Docker, Ansible, Mosquitto, TF Model Optimization Toolkit), and discuss approaches with peers. You must write your own code and your own report.

If asked for a follow-up viva of 15–20 minutes after submission, you must be able to explain any component of your submission and demonstrate it running. Inability to explain your own submitted code will be treated as an integrity concern and referred for investigation.